The Discrete Fourier Transform, Part 6: Cross-Correlation

Douglas Lyon, Ph.D.

Computer Engineering Dept.

Fairfield University, Fairfield CT 06430

lyon@docjava.com

Abstract

This paper is part 6 in a series of papers about the Discrete Fourier Transform (DFT) and the Inverse Discrete Fourier Transform (IDFT). The focus of this paper is on correlation. The correlation is performed in the time domain (slow correlation) and in the frequency domain using a Short-Time Fourier Transform (STFT).  When the Fourier transform is an FFT, the correlation is said to be a “fast” correlation. The approach requires that each time segment be transformed into the frequency domain after it is windowed. Overlapping windows temporally isolate the signal by amplitude modulation with an apodizing function. The selection of overlap parameters is done on an ad-hoc basis, as is the apodizing function selection.

# 1. Introduction to Cross-Correlation

Cross-Correlation (also called cross-covariance) between two input signals is a kind of data processing that enables a means of performing template matching. Correlation typically outputs a scalar variable that denotes a linear metric of similarity between two input signals. Cross-correlation can be done in any number of dimensions. For the purpose of this presentation, we consider one-dimensional normalized cross-correlation between two input signals:

(1)

The coefficient, r, is a measurement of the size and direction of the linear relationship between variables x and y. If these variables move together, where one rises and the other rises at an identical rate, then r = +1. If the other variable does not budge, then r = 0. If the other variable falls at an identical rate, then r = -1. If r is greater than zero, we have positive correlation. If r is less than zero, we have negative correlation.

The sample non-normalized cross-correlation of two input signals requires that r be computed by a sample-shift (time-shifting) along one of the input signals. For the numerator, this is called a sliding dot product or sliding inner product. The dot product is given by:

(2)

When (1) is computed, for all delays, then the output is twice that of the input. It is common to use the pentagon notation when showing a cross correlation:

(3)

Where the asterisk indicates the complex conjugate (a negation of the imaginary part of the number). Input signals should either have the same length, or there should be a policy in place to make them the same (perhaps by zero padding or data replication). If the input signals are real-valued, then we can write:

(4)

Comparing (4) with the convolution:

(5)

Shows that Y is time-reversed before shifting by n. In comparison, correlation has shifting without the time reversal.

# 2. An Example, In Excel

Suppose you are given two signals that have already had their means subtracted. Correlate the signals, without dividing by the standard deviation. The signals are: X=1,2,3,4 with Y = 3, 2, 0, 1.

Figure 1. Y is shifted 7 times

Figure 1 demonstrates that a moving cross correlation requires that the kernel of the signal be shifted so that its leading edge appears and then is shifted until only the trailing edge can be seen. After each shift, the rest of the signal is padded with zeros. The row labeled corr contains the correlation and results from the dot product of X with Y. For example X•Y1=12, X•Y2=17, etc.

# 3. A Slow Cross Correlation

We implement the slow cross correlation using a sliding dot product:

public static double[] shift(double d[], int s) {

double c[] = new double[d.length];

for (int i = 0; i < c.length; i++) {

if ((s + i >= 0)&&(s+i < d.length))

c[i] = d[s + i];

}

return c;

}

public static void main(final String[] args) {

double x[] = {1,2,3,4};

double y[] = {3,2,0,2};

PrintUtils.print(x);

PrintUtils.print(y);

for (int i = -y.length+1; i < y.length; i++){

double slidingY[] = shift(y, i);

PrintUtils.print(slidingY);

System.out.println("dot product:"+Mat1.dot(x,slidingY));

}

}

Where the dot product is implemented using:

static public double dot(final double[] a,

final double[] b) {

final int aLength = a.length;

if (aLength != b.length) {

System.out.println(

"ERROR: Vectors must be of equal length in dot product.");

return 0;

}

double sum = 0;

for (int i = 0; i < aLength; i++) {

sum += a[i] * b[i];

}

return sum;

}

The output matches that given in Figure 1:

1.0 2.0 3.0 4.0

3.0 2.0 0.0 2.0

0.0 0.0 0.0 3.0

dot product:12.0

0.0 0.0 3.0 2.0

dot product:17.0

0.0 3.0 2.0 0.0

dot product:12.0

3.0 2.0 0.0 2.0

dot product:15.0

2.0 0.0 2.0 0.0

dot product:8.0

0.0 2.0 0.0 0.0

dot product:4.0

2.0 0.0 0.0 0.0

The implementation is clearly not optimized, but it is correct and serves to illustrate the sliding dot product nature of the cross correlation.

# 4. A Fast Correlation

A slow implementation of the moving cross correlation algorithm, as shown in Figure 1, will take O(N**2) time. Further, the unoptimized implementation, shown in section 3, has high constant time overhead.

Using the FFT and the correlation theorem, we accelerate the correlation computation. The correlation theorem says that multiplying the Fourier transform of one function by the complex conjugate of the Fourier transform of the other gives the Fourier transform of their correlation. That is, take both signals into the frequency domain, form the complex conjugate of one of the signals, multiply, then take the inverse Fourier transform. This is expressed by:

(6)

We now compare the slow correlation with the fast correlation:

public static void testCorrelation() {

final double x[] = {1,2,3,4};

final double y[] = {3,2,0,2};

PrintUtils.print(x);

PrintUtils.print(y);

System.out.println("cross cor (slow):");

PrintUtils.print(Mat1.slowCorrelation(x, y));

System.out.println("Fast xcor:");

PrintUtils.print(SigProc.correl(x, y, 0));

}

The output follows:

1.0 2.0 3.0 4.0

3.0 2.0 0.0 2.0

cross cor (slow):

12.0 17.0 12.0 15.0 8.0 4.0 2.0

Fast xcor:

12.0 17.0 12.0 15.0 8.0 4.0 2.0

The fast correlation makes use of the FFT, and gets identical results to the slow correlation. So how much faster is the FFT than the slow correlation for small sized arrays? The testing code follows:

public static void main(String[] args) {

for (int i = 1; i < 5; i++) {

System.out.println("size:" + i*256);

speedTestCorrelation(256 * i);

}

//main2(args);

}

public static void speedTestCorrelation(int n) {

final double x[] = new double[n];

final double y[] = new double[n];

StopWatch sw = new StopWatch();

sw.start();

Mat1.slowCorrelation(x, y);

sw.stop();

sw.print("slow correlation is done");

sw.start();

SigProc.correl(x, y, 0);

sw.stop();

sw.print("fast correlation is done");

}

Even for modest arrays, we see substantial speedup (between 2.5 and 12 times faster, for small array sizes):

size:256

slow correlation is done 0.01 seconds

fast correlation is done 0.0040 seconds

size:512

slow correlation is done 0.0060 seconds

fast correlation is done 0.0020 seconds

size:768

slow correlation is done 0.013 seconds

fast correlation is done 0.0030 seconds

size:1024

slow correlation is done 0.025 seconds

fast correlation is done 0.0020 seconds

# 5. Summary

This paper shows how the FFT can be used to speed up cross correlation. Further, it shows that even for small array sizes, substantial speed up can be obtained by using the fast cross correlation. While this is not news [Lyon 97], yet arguments for using the FFT to accelerate the cross correlation are often not supported with specific data on computation time (a situation, which this paper remedies).

The cross correlation has uses in many field of scientific endeavor (music, identification of blood flow, astronomical event processing, speech processing, pattern recognition, financial engineering, etc.).

One of the basic problems with the term normalization when applied to the cross-correlation is that it is defined in different places differently. For example, Pratt suggests that the number of elements in the normalization (an even a square root) is not needed [Pratt]. Lewis suggests using both the square root and the average [Lewis].  Therefore the question of which normalization to use is more an application-specific issue.

# 6. Literature cited

[Lewis] “Fast Normalized Cross-Correlation” by J.P. Lewis, http://www.idiom.com/~zilla/Papers/nvisionInterface/nip.html last accessed 8/24/09.

[Lyon 97] Java Digital Signal Processing, Douglas A. Lyon and H. Rao, M&T Press (an imprint of Henry Holt). November 1997.

[Pratt] W. Pratt, Digital Image Processing, John Wiley, New York, 1978.